Computer Systems Organization
CSCI 463

Fall 2024

Homework #9

1. For the following problems, assume that cache and main memory are checked sequentially (not in parallel) and that no data is updated in cache (so no data needs to be written back to main memory). Given those assumptions, you can use the formula in the slides to calculate EAT.

a) Suppose a system has main memory with an access time of 100ns, a cache with a 20ns access time and a hit rate of 99%. What is the EAT?

b) Same as above with a hit rate of 98%.

c) Same as above with a hit rate of 95%.

2. Relationship between normalized and denormalized number.

a) For a given standard, the smallest normalized number is approximately equal to the largest denormalized number. Why?

b) If you don’t round off, which of the two is always larger?

3. Consider the following piece of code. The answer sheet contains the assembler’s symbol table in the form of a chart. (Although you can see all the columns, note that in real life an entry would not be added to the symbol table until the first time the variable appears in the code.)

The rows show the value of the entries after the execution of each line of code. Leave the entry for a variable blank if the variable has not been added to the symbol table yet. Enter 000 (using an apostrophe if necessary to allow initial zeroes) if the entry has been added but has not been given a value yet. Once the assembler has given the variable a value, enter that value.

I have filled in the first two lines for you. I have also completed the first column so that you can see the benefit of a two-pass assembler. Your job is to fill in the rest of the chart.

100       LOAD Addr		Initialize Next with

101       STORE Next		  starting address

102       LOAD Num		Initialize Ctr

103       ADD  Num1

104       STORE Ctr

105 Loop  LOAD Sum

106       SUBT Num1

107       STORE Sum

108       JUMP Loop

109 Addr  HEX 114

110 Next  HEX 0

111 Num   DEC 5

112 Sum   DEC 16

113 Ctr   HEX 0

114 Num1  DEC 10